## LOAD KEY

## Response

| Field | Value | Remark |
| --- | --- | --- |
| SW | 9000 on success | See specific error codes below; other values are documented in Status Words |

### Specific error codes

| SW | Reason |
| --- | --- |
| 6300 | Secure transmission failed (DataINSECURE is invalid) |
| 6382 | PICC key not supported (possible root reason: incompatible entry) |
| 6383 | Host key not supported(possible root reason: incompatible entry, key type is not AES) |
| 6384 | Plain transmission not supported (possible root reason: secure transmission enforced by configuration) Plain transmission not allowed for a host key |
| 6385 | Secure transmission not supported (possible root reason: the host key index specified in bits 3-0 of P1 is not allowed) |
| 6386 | Volatile memory is not available A host key can't be loaded into the volatile memory |
| 6387 | Non volatile memory is not available (the device does not have a Secure Element) |
| 6388 | Key index is not valid Key index is not allowed for the key type |
| 6389 | Key length is not valid Key length does not match the key type |

## GENERAL AUTHENTICATE

## Response

| Field | Value | Remark |
| --- | --- | --- |
| SW | 9000 on success | See specific error codes below; other values are documented in Status Words |

### Specific error codes

| SW | Reason |
| --- | --- |
| 6300 | General error |
| 6382 | Security status not satisfied - A host/device authentication must be performed first |
| 6383 | Authentication with the PICC failed |
| 6384 | The selected key is not valid for this type of PICC |
| 6386 | The type of key is not supported |
| 6388 | Key index is not valid The selected key is not valid for this PICC |
| 6389 | Key length is not valid The selected key is not valid for this PICC |
| 6581 | The key addressed by P1-P2 does not exist |